North Lincolnshire Council

Daily Event Analysis

5th October 2010

# Overview

Low Santon measured a daily mean PM10 concentration of 76µg/m3 on Tuesday the 5th October 2010 exceeding the allowed 50µg/m3 as a daily mean. A check of the data has revealed that concentrations of PM10 began to increase at 12:00 and fell at 18:00. During this 6 hour period concentrations at the Low Santon TEOM remained at over 100µg/m3. The Dawes Lane Coke Oven (DLCO) Osiris measured a daily mean concentration of 42µg/m3. The Osiris did not measure the same elevated levels that the Low Santon TEOM witnessed between 12:00 & 18:00

# Met Conditions

Consistent SW winds were observed throughout the day. The 6 hour period of focus ranged between 205° and 236°. Wind speeds were relatively low averaging at 2.2 m/s. The period of focus averaged at 2.7m/s. The highest wind speed in this period was 3.2m/s in which the concentration of PM10 reached 205µg/m3. No rainfall was measured throughout the day.

# Episode Wind Range

# Potential Sources

The above map shows the Low Santon Monitoring Station (indicated by the red lines) and the DLCO Osiris (indicated by the green). The black line within the red shows the wind direction of the highest concentration during the 6 hour period of focus.

As the Osiris was not subject to the elevated levels at the TEOM it is likely that the source of the episode did not originate from within the wind sectors relevant to the Osiris (green lines). By removing the area of the TEOM wind sectors also affecting the Osiris we can identify specific areas on site that may have been responsible for the exceedance.

The likely areas include;

- The Slag Haul Road

- The Redbourne Steel Stocking Area

- The Intermediate Site

The black line indicates the highest measured concentration during this period. This runs along the Slag Haul Road and through the Redbourne Steel Stocking Area.