North Lincolnshire Council

Daily Event Analysis

8th October 2010

# Overview

The Dawes Lane Coke Oven (DLCO) Osiris measured a daily mean PM10 concentration of 62µg/m3 on Friday the 8th October 2010 exceeding the allowed 50µg/m3 as a daily mean. A check of the data has revealed that concentrations of PM10 were above the objective throughout the day. During this 24 hour period concentrations at the Dawes Lane Coke Oven Osiris remained at around 60µg/m3. The Low Santon TEOM measured a daily mean concentration of 36µg/m3. The TEOM did not measure the same elevated levels that the Coke Oven Osiris witnessed at any point during the day.

# Met Conditions

Consistent E winds were observed throughout the day. The wind for the 24hr period ranged between 82° and 113°. Wind speeds were slightly higher than the previous few days averaging at 3.9 m/s. The highest wind speed of the day was 5.2m/s in which the concentration of PM10 reached 83µg/m3. No rain was measured throughout the day.

# Episode Wind Range

# Potential Sources

The above map shows the DLCO Osiris (indicated by the red lines) and the Low Santon TEOM (indicated by the green). The black line within the red shows the wind direction of the highest concentration during the day.

As the TEOM was not subject to the same elevated levels at the Osiris, it is likely that the source of the episode did not originate from within the wind sectors relevant to the TEOM (green lines). By removing the area of the Osiris wind sectors also affecting the TEOM we can identify specific areas on site that may have been responsible for the exceedance.

The likely areas include;

- Tarmac

- Slag Haul Road Entrance

The black line indicates the highest measured concentration during this period. This runs through the Tarmac site.