North Lincolnshire Council

Daily Event Analysis

12th October 2010

# Overview

The Amvale Osiris measured a daily mean PM10 concentration of 59µg/m3 on Tuesday the 12th October 2010 exceeding the allowed 50µg/m3 as a daily mean. A check of the data has revealed that concentrations of PM10 had an extended period of elevated concentrations after 19:00. During this period concentrations at the Amvale Osiris remained at between 60 & 80µg/m3. The Redbourne TEOM measured a daily mean concentration of 19µg/m3 and the East Common Lane TEOM 28µg/m3. Neither TEOM site recorded the elevated levels after 19:00 witnessed by the Amvale Osiris.

# Met Conditions

Consistent NE winds were observed throughout the day. The wind after 19:00 ranged between 334° and 57°. Wind speeds averaged at 2m/s during the period of focus. The highest wind speed in this period was 2.3m/s in which the concentration of PM10 reached 90µg/m3. No rain was measured throughout the day.

# Episode Wind Range

# Potential Sources

The above map shows the Amvale Monitoring Station (indicated by the red lines) and the East Common Lane & Redbourne Sites (indicated by the green). The black line within the red shows the wind direction of the highest concentration during the period of focus.

As the TEOMs were not subject to the same elevated levels as the Osiris, it is likely that the source of the episode did not originate from within the wind sectors relevant to the TEOMs (green lines). By removing the area of wind sectors affecting both the Osiris & TEOM we can identify specific areas on site that may have been responsible for the exceedance.

The likely areas include;

- Civil & Marine

- Sinter Plant

- Ore Beds

- Intermediate Site

- Blast Furnaces

The black line indicates the highest measured concentration during this period. This runs through the Blast Furnaces.