North Lincolnshire Council

Daily Event Analysis

30th October 2010

# Overview

The Low Santon TEOM measured a daily mean PM10 concentration of 56µg/m3 on Saturday 30th October 2010 exceeding the allowed 50µg/m3 as a daily mean. The Dawes Lane Coke Oven (DLCO) Osiris measured a daily PM10 concentration of 40g/m3. A check of the data has revealed that the PM10 concentrations were significantly above the 50g/m3 threshold between 11:30 and 17:00 at the Low Santon TEOM, the same period at the DLCO Osiris measured concentrations marginally above the daily objective limit. The average PM10 concentration for this time period at the Low Santon TEOM was 119g/m3. The DLCO Osiris averaged at 60g/m3.

# Met Conditions

Consistent SW winds were observed throughout the day. The wind within the 5 hour & 30 min period of focus ranged between 193° and 228°. Wind speeds averaged at 2.6 m/s. Light rain was measured for a short period late in the afternoon.

# Episode Wind Range

# Potential Sources

The above map shows the Low Santon TEOM (indicated by the red lines) and the DLCO Osiris (indicated by the green). The black line within the red shows the wind direction of the highest concentration recorded during the 5 hour period of focus.

As the Low Santon TEOM measured consistently higher concentrations during this period, it is likely that the TEOM was affected by a different source to the DLCO Osiris. Therefore, the areas in which the red and green lines intersect are unlikely to be the location of the problem.

The likely areas include;

- Tarmac

- Redbourn Site

- Slag Haul Road

- Intermediate Site

The black line indicates the highest measured concentration of 191g/m3 recorded at the Low Santon TEOM during this period. It runs along the Slag Haul Road and the Intermediate Site.

The 27th, 28th and now the 30th October have shown an almost identical pattern characterised by an increase around 12:00 returning to compliant levels of PM10 around 17:00. Could the sites identified please check their activity data?