North Lincolnshire Council

Daily Event Analysis

31st October 2010

# Overview

The Amvale Osiris measured a daily mean PM10 concentration of 65µg/m3 on Sunday 31st October 2010 exceeding the allowed 50µg/m3 as a daily mean. Nearby Scunthorpe sites, East Common Lane & Redbourn Club did not exceed the Daily Mean Objective. A check of the data has revealed that the PM10 concentrations were significantly above the 50g/m3 threshold between 07:00 and 09:30 at the Amvale Osiris. The average PM10 concentration for this time period at Amvale was 156g/m3.

# Met Conditions

Winds varied throughout the day ranging from NW to SE. The wind within the 2 hour & 30 minute period of focus ranged between 357° and 52°. Wind speeds averaged at 1.7 m/s. Light rain was measured in the morning, heavy rain was observed late afternoon.

# Episode Wind Range

# Potential Sources

The above map shows the Amvale Osiris (indicated by the red lines) and the neighbouring Scunthorpe sites of East Common Lane & Redbourn Club (indicated by the green). The black line within the red shows the wind direction of the highest concentration recorded during the 2 hour & 30 minute period of focus.

As the Amvale Osiris measured consistently higher concentrations during this period, it is likely that the Osiris was affected by a different source to the neighbouring TEOMs. Therefore, the areas in which the red and green lines intersect are unlikely to be the location of the problem.

The likely areas include;

- The Sinter Plant

- Civil & Marine

- Ore Beds

- Intermediate Site

- Blast Furnaces

The black line indicates the highest measured concentration of 276 g/m3 recorded at the Amvale Osiris during this period. It runs through the Blast Furnaces.